WARNING: If you setup wrong configuration, system maybe no action

when you rebooting your system. Then power off your system,

and then press the "INS" key after you power on the system

immediately to clear extended CMOS setup. you can re-setup

your system now.

The "QUICKSET" is a NEAT EXTENDED CMOS setup utility, there are

four topics :

1. SET MEMORY TYPE AND SIZE.

2. SET MEMORY WAIT STATE.

3. SET EMS MEMORY.

4. SET SHADOW MEMORY.

First, press "QUICKSET" entering extended CMOS utility, then displays

following main menu:

NEAT QUICK SETUP UTILITY Vx.x

1. SET MEMORY TYPE AND SIZE.

2. SET MEMORY WAIT STATE.

3. SET EMS MEMORY.

4. SET SHADOW MEMORY.

5. WRITE TO CMOS AND REBOOT.

6. EXIT.

# # : Moves items,   Enter : Select,   ESC : Abort

Please select it sequentially.

Use "#" or "#" key to move items.

Use "Enter" key to select items.

Use "ESC" key to exit to system, but the CMOS data no change.

For example:

Total memory 1 Mbyte with 41256 DRAM run in 16 MHz 0 wait state,

the setup procedure is:

1. Move item to "SET MEMORY TYPE AND SIZE", and press ENTER

then displays following message:

DRAM TYPE             TOTAL

BANK0   BANK1   BANK2   BANK3   MEMORY    INTERLEAVE

------------------------------------------------------------

1.     256K    0       0       0       512KB

2.     1M      0       0       0       2MB

3.     256K    64K     0       0       640KB

4.     256K    256K    0       0       1MB            Y

5.     1M      1M      0       0       4MB            Y

6.     256K    256K    256K    0       1.5MB

7.     256K    256K    1M      0       3MB

8.     1M      1M      1M      0       6MB

9.     256K    64K     256K    256K    1.64MB

10.    256K    256K    256K    256K    2MB            Y

11.    256K    64K     1M      1M      4.64MB

12.    256K    256K    1M      1M      5MB            Y

13.    1M      1M      1M      1M      8MB            Y

2. Use "#" key move item to

"256K    256K    0       0       1MB            Y",

then press "ENTER" to select it.

The screen will return to main menu.

3. Move item to "SET MEMORY WAIT STATE", and press ENTER

then displays following message:

SYSTEM SPEED IS:",

"1. 16MHz",

"2. 12MHz",

"3. 10MHz",

4. Use "#" key move item to "16MHz", then press "ENTER"

to select it. Then displays following message:

INTERLEAVE 0 WAIT STATE, Lowest DRAM request is 100NS",

INTERLEAVE 1 WAIT STATE, Lowest DRAM request is 120NS",

NON-INTERLEAVE 0 WAIT STATE, Lowest DRAM request is  60NS",

NON-INTERLEAVE 1 WAIT STATE, Lowest DRAM request is  80NS",

5. Use "#" key move item to

"INTERLEAVE 0 WAIT STATE, Lowest DRAM request is 100NS",

then press "ENTER" to select it.

The screen will return to main menu.

6. Move item to "SET EMS MEMORY", and press ENTER

then displays following message:

1. Disable LIM EMS",

2. Enable  LIM EMS",

7. Use "#" key move item to "Disable LIM EMS",

then press "ENTER" to select it.

The screen will return to main menu.

8. Move item to "SET SHADOW MEMORY", and press ENTER

then displays following message:

ROM BIOS",

1. Disable",

2. Enable",

9. If you select "Disable" then the total memory can test

to 1024K, otherwise, the memory can test to 640k only.

Because the memory in 384k reserve for SHADOW RAM use.

After selection, the screen will return to main menu.

NOTE: If your total memory size is over 1M bytes, the 384K

memory locate in 640K to 1024k must to be reserve for

SHADOW RAM use, so the total memory size test will

lose the 384K.

10. Use "#" key move item to "WRITE TO CMOS AND REBOOT.",

then press "ENTER", the system will write the setuped

data to CMOS then rebooting system.

11. Use "#" key move item to "EXIT", then press "ENTER",

or press "ESC" ket to return DOS system. CMOS data

will no change.

If you want to set EMS function then the step 7 will change to following:

7. Use "#" key move item to "Enable LIM EMS",

then press "ENTER" to select it. Then displays EMS menu:

1. Select I/O port address for EMS use

2. Set starting Address for EMS use

3. Set EMS Page 0 Address Extention

4. Set EMS Page 1 Address Extention

5. Set EMS Page 2 Address Extention

6. Set EMS Page 3 Address Extention

7. Select total Memory Size for EMS

7-1. Use "#" key move item to "Select I/O port address for EMS use",

then press "ENTER" to select it. Then displays:

1. 208H/209H

2. 218H/219H

3. 258H/259H

4. 268H/269H

5. 2A8H/2A9H

6. 2B8H/2B9H

7. 2E8H/2E9H

7-1-1. Use "#" key move item to "208H/209H",

then press "ENTER" to select it.

The screen will return to EMS menu.

7-2. Use "#" key move item to "Set starting address for EMS use",

then press "ENTER" to select it. Then displays:

1. C0000H - CFFFFH

2. C4000H - D3FFFH

3. C8000H - D7FFFH

4. CC000H - DBFFFH

5. D0000H - DFFFFH

6. D4000H - E3FFFH

7. D8000H - E7FFFH

8. DC000H - EBFFFH

9. E0000H - EFFFFH

7-2-1. Use "#" key move item to "D0000H - DFFFFH",

then press "ENTER" to select it.

The screen will return to EMS menu.

7-3. Use "#" key move item to "Set EMS Page 0 Address Extention

then press "ENTER" to select it. Then displays:

1. 1 Mbyte to 2 Mbyte

2. 2 Mbyte to 4 Mbyte

3. 4 Mbyte to 6 Mbyte

4. 6 Mbyte to 8 Mbyte

7-3-1. Use "#" key move item to "1 Mbyte to 2 Mbyte",

then press "ENTER" to select it.

The screen will return to EMS menu.

7-4. Use "#" key move item to "Set EMS Page 1 Address Extention

then press "ENTER" to select it. Then displays:

1. 1 Mbyte to 2 Mbyte

2. 2 Mbyte to 4 Mbyte

3. 4 Mbyte to 6 Mbyte

4. 6 Mbyte to 8 Mbyte

7-4-1. Use "#" key move item to "1 Mbyte to 2 Mbyte",

then press "ENTER" to select it.

The screen will return to EMS menu.

7-5. Use "#" key move item to "Set EMS Page 2 Address Extention

then press "ENTER" to select it. Then displays:

1. 1 Mbyte to 2 Mbyte

2. 2 Mbyte to 4 Mbyte

3. 4 Mbyte to 6 Mbyte

4. 6 Mbyte to 8 Mbyte

7-5-1. Use "#" key move item to "1 Mbyte to 2 Mbyte",

then press "ENTER" to select it.

The screen will return to EMS menu.

7-6. Use "#" key move item to "Set EMS Page 3 Address Extention

then press "ENTER" to select it. Then displays:

1. 1 Mbyte to 2 Mbyte

2. 2 Mbyte to 4 Mbyte

3. 4 Mbyte to 6 Mbyte

4. 6 Mbyte to 8 Mbyte

7-6-1. Use "#" key move item to "1 Mbyte to 2 Mbyte",

then press "ENTER" to select it.

The screen will return to EMS menu.

7-7. Use "#" key move item to "Select total Memory Size for EMS

then press "ENTER" to select it. Then displays:

1. 0.5 MB

2. 1 MB

3. 2 MB

4. 3 MB

5. 4 MB

6. 5 MB

7. 6 MB

8. 7 MB

7-7-1. Use "#" key move item to "0.5 MB",

then press "ENTER" to select it.

The screen will return to EMS menu.

Press "ESC" key return to main menu.

Note: If you has enable EMS, you must copy the "emm.sys" file to the

boot-up disk, and add 1 line "device=emm.sys" in config.sys file.